SUMMARY OF STEPS OF LAB NO 3

Lab 3 — Series Circuit & Multimeter Basics

Vs = 5V (Task 1) & 9V (Task 2) ·

Tools: DMM

breadboard

power supply

Resistance measurement

1

Turn off power and isolate the component to be tested.

2

Plug probes into the correct jacks (same jacks as voltage). Turn the knob to Ohms (Ω), start at the lowest range.

3

Touch probe tips together — display should read ~0. Then connect across the component and read the value. If OL, move to a higher range.

Voltage measurement

Set knob to DC Volts. Plug red probe into VΩ jack, black into COM.

Connect probes in parallel (across) the component or source.

Read the display. If a minus sign appears, reverse the probes.

Current measurement

Turn off power. Break the circuit at the point where current is to be measured.

Set knob to DC Amps / mA. Insert the meter in series at the break. Never connect in parallel.

Turn power on, read the display, then power off and restore the circuit.

Lab tasks performed

T1

Built series circuit (5V, R1 = 5.1 kΩ, R2 = 3.3 kΩ). Measured V across each resistor and current between them. Verified with KVL.

T2

Built series circuit (9V, R1 = 3 kΩ, R2 = 10 kΩ, R3 = 5 kΩ). Measured voltages and confirmed same current at nodes 2 and 3.