Circuit Analysis (Lab 7)

Experiment 7: Analysis of Operational Amplifiers Operations

CLO4 Construct amplifiers with operational amplifiers. Psychomotor / Affective ¦ Level 4 (6-7)

Name: ABEERA                                                            REGISTERATION NUMBER: 2025-CE-66

LAB NO: 7

| Psychomotor/ Affective | Level 1 (1) | Level 2 (2-3) | Level 3 (4-5) | Level 4 (6-7) | Report Marks (3) | Total (10) |
| --- | --- | --- | --- | --- | --- | --- |
| CLO4 — Op-Amp Amplifiers |  |  |  |  |  |  |

Objective

Objective

| Pin | Name / Function |
| --- | --- |
| 1 | Offset Null |
| 2 | Inverting Input Terminal (−) |
| 3 | Non-Inverting Input Terminal (+) |
| 4 | Negative Voltage Supply (−VCC) |
| 5 | Offset Null |
| 6 | Output Voltage |
| 7 | Positive Voltage Supply (+VCC) |

objective of this lab is to investigate the input to output relationship for inverting and non-inverting amplifiers using the IC 741 operational amplifier.

Apparatus Required

Apparatus Required

| Sr. No. | Apparatus | Range | Quantity |
| --- | --- | --- | --- |
| 1 | Regulated Power Supply | Standard | 1 |
| 2 | Signal Generator | Standard | 1 |
| 3 | Oscilloscope | Standard | 1 |
| 4 | Digital Multimeter | Standard | 1 |
| 5 | Breadboard & Wires | Standard | As Required |
| 6 | Resistors | 1kΩ, 2kΩ, 3kΩ | 3 |
| 7 | IC 741 Op-Amp | Standard | 1 |

Description

Description

Operational amplifiers can be used in two basic configurations to create amplifier circuits. One is the inverting amplifier where the output is the inverse or 180° out of phase with the input, and the other is the non-inverting amplifier where the output is in the same sense or in phase with the input.

Both operational amplifier circuits are widely used and find applications in different areas. The IC 741 op-amp is a general-purpose device powered from a dual supply (±12 V).

Pin Configuration of IC 741 Op-Amp

- Voltage control knob should be kept at minimum position.

- Current control knob of RPS should be kept at maximum position.3. Carefully utilize the oscilloscope channels and function generator.

- Verify polarity of dual power supply (Pin 7 → +VCC, Pin 4 → −VCC) before powering the IC.

- Do not exceed the IC 741 maximum supply voltage (±18 V).

Procedure for Inverting Amplifier

Procedure for Inverting Amplifier

- Wire up the circuit as shown in the figure below on the breadboard.

- Give the input (40 Hz, 2 V p-p sine wave) to the inverting terminal of the op-amp and simultaneously display the input and output on the oscilloscope.

| 8 | No Connection (NC) |
| --- | --- |

Precautions

40

Hz

2

V p-p

R1

k

1

Ω

R2

k

3

Ω

-

+

741

OUTPUT

Inverting Amplifier   Av = -R2/R1 = -3

40

Hz

V p-p

2

-

+

741

R1

k

1

Ω

R2

2

k

Ω

OUTPU

Non-Inverting Amplifier   Av = 1+R2/R1 = +3

Procedure for Non-Inverting Amplifier

Draw the input and output waveforms in the Result section.

- Wire up the circuit as shown in the figure above on the breadboard.

- Give the input to the non-inverting terminal of the op-amp and simultaneously display the input and output on the oscilloscope.

- Draw the input and output waveform in the Result section.

Result

Result

Figure 7: Input Voltage Signal

Vin  1V pp  40 Hz  (solid)

Figure 8: Inverted Output

Vin  1V pp  (solid)

Vout  3V pp  180 deg  (dashed)

Figure 9: Non-Inverting Output

Vin  1V pp  (solid)

Vout  3V pp  0 deg  (dashed)

Parameter

Inverting Amplifier

Non-Inverting Amplifier

Figure 7: Input Voltage Signal

Vin  1V pp  40 Hz  (solid)

Figure 8: Inverted Output

Vin  1V pp  (solid)

Vout  3V pp  180 deg  (dashed)

Figure 9: Non-Inverting Output

Vin  1V pp  (solid)

Vout  3V pp  0 deg  (dashed)

Parameter

Inverting Amplifier

Non-Inverting Amplifier

| Input Voltage (Vin p-p) | 1 V | 1 V |
| --- | --- | --- |
| Output Voltage (Vout p-p) | 3 V | 3 V |
| Voltage Gain (Av) | 3 | 3 |
| Phase Shift | 180° (inverted) | 0° (in-phase) |
| Frequency | 40 Hz | 40 Hz |
| Gain Formula | Av = -R2/R1 = -3 | Av = 1+R2/R1 = +3 |

Post Lab Questions

Post Lab Questions

| 1 | Define inverting amplifier. |
| --- | --- |
| An inverting amplifier is an op-amp circuit where the input signal is applied to the inverting (−) terminal through resistor R1, with feedback resistor R2 connected from the output back to the inverting terminal. The output is 180° out of phase (inverted) with the input — when input goes positive, output goes negative. Gain: Av = −R2 / R1 (negative sign = phase inversion) → For this lab: Av = −3kΩ/1kΩ = −3 |  |
| 2 | Define non-inverting amplifier. |
| A non-inverting amplifier is an op-amp circuit where the input signal is applied directly to the non-inverting (+) terminal. The output is in phase (0° phase shift) with the input. The gain is always ≥ 1. Gain: Av = 1 + R2 / R1 (always positive) → For this lab: Av = 1 + 2kΩ/1kΩ = +3 |  |
| 3 | What are the applications of inverting amplifiers? |
| - Summing / audio mixer: multiple inputs combined at the inverting terminal. - Integrator circuit: R2 replaced by a capacitor; used in analog computing and PID controllers. - Sign inverter / polarity changer: provides 180° phase flip in signal chains. - Active filters: multiple-feedback low-pass and band-pass filter topologies. |  |
| 4 | What are the applications of non-inverting amplifiers? |
| - Voltage buffer / unity-gain follower: high input impedance, low output impedance — isolates circuit stages. - Instrumentation amplifier: high Zin minimises loading on sensors (thermocouples, strain gauges). - Audio preamplifier: preserves phase of audio signals. - Active filters: Sallen-Key topology for Butterworth/Chebyshev responses. |  |
| 5 | What is gain of an amplifier? |
| Voltage gain (Av) is the ratio of output amplitude to input amplitude: Av = Vout / Vin Gain > 1 → amplification. Gain < 1 → attenuation. Negative Av → phase inversion. In decibels: AdB = 20 × log10(¦Av¦). For this lab: ¦Av¦ = 3 → 20 × log10(3) ≈ 9.54 dB |  |
| 6 | How do you calculate the gains for inverting and non-inverting amplifiers? |
| Inverting amplifier: Av = −R2 / R1 Lab values: Av = −3 kΩ / 1 kΩ = −3 The negative sign indicates 180° phase inversion. R1 sets input impedance; R2 sets the feedback level. Non-inverting amplifier: Av = 1 + R2 / R1 Lab values: Av = 1 + 2 kΩ / 1 kΩ = +3 Gain is always ≥ 1. R1 and R2 form a voltage divider feeding back to the inverting terminal to stabilise gain. |  |