LAB 11 SUMMARY

The primary objective of this experiment is to verify Norton's Theorem practically by constructing the original circuit on a breadboard, measuring the load current directly, then building the Norton equivalent circuit and confirming that the load current from both circuits is identical. A secondary objective is to develop the skill of simplifying complex linear circuits into their Norton equivalent form for easier analysis.

2. APPARATUS USED

| Sr. No. | Apparatus | Specification | Quantity |
| --- | --- | --- | --- |
| 1 | Regulated Power Supply (RPS) | 0–30V DC | 1 |
| 2 | Signal Generator | Standard | 1 |
| 3 | Oscilloscope | Standard | 1 |
| 4 | Digital Multimeter (DMM) | Standard | 1 |
| 5 | Breadboard & Connecting Wires | Standard | As required |
| 6 | Resistors | 330Ω × 3, 1kΩ × 1 | 4 |
| 7 | Potentiometer (DRB) | 10kΩ | 1 |

3. THEORY AND BACKGROUND

3.1 Statement of Norton's Theorem

Norton's Theorem states that any linear, bilateral electrical circuit — no matter how complex — can be reduced and replaced by a simple equivalent circuit consisting of:

- A single constant current source (IN) in parallel with

- A single equivalent resistance (RN)

connected across the load terminals. This equivalent circuit behaves identically to the original circuit as far as any external load connected to those terminals is concerned.

3.2 Key Parameters

Norton Current (IN): IN is defined as the short-circuit current that flows through the load terminals when they are short-circuited (i.e., a wire of zero resistance is connected between them). To find IN experimentally, the load resistor is removed and the terminals are shorted with a wire, and the current through that short is measured with an ammeter.

Norton Resistance (RN): RN is the equivalent resistance seen looking back into the circuit from the load terminals, after all independent sources have been deactivated:

- Voltage sources → replaced by short circuits (wires)

- Current sources → replaced by open circuits (removed)

RN is measured using an ohmmeter across the open load terminals with the source deactivated, or calculated analytically.

Load Current (IL): Once IN and RN are known, the load current through any load resistance RL can be found using the current divider formula:

IL = IN × RN / (RN + RL)

3.3 Relationship to Thevenin's Theorem

Norton's Theorem is the dual of Thevenin's Theorem. The two are directly related:

- RN = RTH (the Norton and Thevenin resistances are always equal)

- IN = VTH / RTH (Norton current = Thevenin voltage ÷ Thevenin resistance)

- VTH = IN × RN (Thevenin voltage = Norton current × Norton resistance)

This means any circuit that can be represented by a Thevenin equivalent can equally be represented by a Norton equivalent, and vice versa — they are two different but equivalent ways of looking at the same circuit behavior.

4. CIRCUIT DESCRIPTION

4.1 Original Circuit (Circuit-I)

The original circuit consists of:

- RPS voltage source E = 10V DC

- R1 = 330Ω in series from the positive terminal of the source

- R3 = 330Ω connected from the mid-node (between R1 and R2) to ground — acting as a shunt resistor

- R2 = 330Ω in series from the mid-node toward the load terminals

- RL = 1kΩ load resistor connected at the output terminals

- An ammeter (0–30mA range) connected in series with RL to measure IL

4.2 To Find IN Circuit

Same circuit but RL is removed and the load terminals are shorted with a wire. The ammeter now reads the short-circuit current IN directly.

4.3 To Find RN Circuit

The RPS is short-circuited (replaced by a wire). RL is removed. An ohmmeter is connected across the open load terminals to read RN directly.

4.4 Norton's Equivalent Circuit

The Norton equivalent is implemented using:

- RPS set as a constant current source (using a 10kΩ series resistor to approximate a current source behavior)

- A DRB (Decade Resistance Box) set to RN = 495Ω in parallel

- Load RL = 1kΩ connected at the output terminals

- Ammeter in series with RL to measure IL from the equivalent

5. STEP-BY-STEP PROCEDURE

Step 1: Set the RPS voltage to zero. Assemble the original Circuit-I on the breadboard as per the circuit diagram. Connect R1, R2, R3, and RL with the ammeter in series with RL.

Step 2: Switch on the RPS and set E = 10V. Note down the ammeter reading — this gives IL for Circuit-I.

Step 3 (Find IN): Switch off the RPS. Remove RL from the circuit. Connect a short-circuit wire across the load terminals. Switch RPS back on to E = 10V. The ammeter now reads IN (Norton current).

Step 4 (Find RN): Switch off the RPS. Remove it from the circuit and replace it with a short-circuit wire. Remove the short across the load terminals. Connect the ohmmeter/multimeter in resistance mode across the open load terminals. Read the resistance — this is RN.

Step 5 (Equivalent Circuit): Build the Norton equivalent circuit. Set the constant current source to deliver IN = 10.10 mA. Set the DRB to RN = 495Ω. Connect RL = 1kΩ as the load. Measure IL from this equivalent circuit — it must match Circuit-I.

Step 6: Compare both IL values. If they match (within measurement tolerance), Norton's Theorem is verified.

6. CALCULATIONS IN DETAIL

Given: E = 10V, R1 = R2 = R3 = 330Ω, RL = 1kΩ

6.1 Finding IN (Norton Short-Circuit Current)

When load terminals are shorted, R2 and R3 are both connected from node-A to ground, so they are in parallel:

R₂ ∥ R₃ = (330 × 330) / (330 + 330) = 108,900 / 660 = 165 Ω

Total circuit resistance seen by the source:

R_total = R1 + (R2 ∥ R3) = 330 + 165 = 495 Ω

Total current from source:

I_total = E / R_total = 10 / 495 = 20.20 mA

Voltage at node-A (junction of R1, R2, R3):

V_A = E × (R2 ∥ R3) / R_total = 10 × 165 / 495 = 3.333 V

Norton current IN (current through the shorted R2 branch):

IN = V_A / R2 = 3.333 / 330 = 10.10 mA

6.2 Finding RN (Norton Equivalent Resistance)

With the voltage source shorted (E = 0, replaced by wire), looking into the load terminals:

- R1 and R3 are both connected between node-A and ground → they are in parallel

R1 ∥ R3 = (330 × 330) / (330 + 330) = 165 Ω

- This parallel combination is in series with R2 (since R2 connects from node-A to the load terminal):

RN = R2 + (R1 ∥ R3) = 330 + 165 = 495 Ω

6.3 Load Current IL from Original Circuit (Node Analysis)

Applying KCL at node-A (voltage = VA) and node-B (voltage = VB):

At node-B: Current through R2 = Current through RL

(VA − VB) / R2 = VB / RL (VA − VB) / 330 = VB / 1000 VA = VB × (1 + 330/1000) = VB × 1.33

At node-A: Current into node-A = Currents out

(E − VA) / R1 = VA / R3 + (VA − VB) / R2

Substituting VA = 1.33 VB and E = 10:

(10 − 1.33VB) / 330 = 1.33VB / 330 + VB / 1000

Solving:

10/330 = (2 × 1.33VB)/330 + VB/1000 0.03030 = 0.008061VB + 0.001VB = 0.009061VB VB = 3.3445 V

Load current in original circuit:

IL (Circuit-I) = VB / RL = 3.3445 / 1000 = 3.34 mA

6.4 Load Current IL from Norton Equivalent Circuit

Using the current divider on the Norton equivalent (IN in parallel with RN, driving load RL):

IL (Equivalent) = IN × RN / (RN + RL) IL = 10.10 × 10⁻³ × 495 / (495 + 1000) IL = 10.10 × 10⁻³ × 495 / 1495 IL = 10.10 × 10⁻³ × 0.3311 IL = 3.34 mA ✅