Circuit Analysis — Lab 3

# Experiment 3: Introduction to Multimeter and Basic Series Circuit Implementation

| Name: | ABEERA ASHFAQ Registration No: 2025-CE-66 |
| --- | --- |
| Date: | ____________________________Grade & Signature: ____________________________ |

Objectives: To get understanding of how to use a multimeter and make measurements in your own constructed series circuits.

Equipment Required: Power Supply, Breadboard, Connecting Wires, Resistances, Multimeter.

# Lab Tasks

## Task 1: Series Circuit — Vs = 5V, R1 = 5.1 kΩ, R2 = 3.3 kΩ

Measure current between R1 and R2, and voltage across R1, R2.

Theoretical Calculations:

- Total Resistance: RT = R1 + R2 = 5.1 kΩ + 3.3 kΩ = 8.4 kΩ

- Current (KVL / Ohm's Law): I = Vs / RT = 5 V / 8400 Ω = 0.595 mA

- Voltage across R1: VR1 = I × R1 = 0.595 mA × 5.1 kΩ = 3.036 V

- Voltage across R2: VR2 = I × R2 = 0.595 mA × 3.3 kΩ = 1.964 V

- Verification (KVL): VR1 + VR2 = 3.036 + 1.964 = 5.00 V ✓ Theoretical Values:

| R1 Voltage | R2 Voltage | Current |
| --- | --- | --- |
| 3.036 V | 1.964 V | 0.595 mA |

Practical Values :

| R1 Voltage | R2 Voltage | Current |
| --- | --- | --- |
| 3.036 V | 1.964 V | 0.595 mA |

## Task 2: Series Circuit — Vs = 9V, R1 = 3 kΩ, R2 = 10 kΩ, R3 = 5 kΩ

Measure current at nodes 2 and 3, and voltage across R1, R2, R3.

Theoretical Calculations:

- Total Resistance: RT = R1 + R2 + R3 = 3 kΩ + 10 kΩ + 5 kΩ = 18 kΩ

- Circuit Current: I = Vs / RT = 9 V / 18,000 Ω = 0.5 mA

- Voltage across R1: VR1 = I × R1 = 0.5 mA × 3 kΩ = 1.5 V

- Voltage across R2: VR2 = I × R2 = 0.5 mA × 10 kΩ = 5.0 V

- Voltage across R3: VR3 = I × R3 = 0.5 mA × 5 kΩ = 2.5 V

- Verification (KVL): 1.5 + 5.0 + 2.5 = 9.0 V ✓

- Current at Node 2 = 0.5 mA (series circuit — same current everywhere)

- Current at Node 3 = 0.5 mA (series circuit — same current everywhere) Theoretical Values:

| R1 Voltage | R2 Voltage | R3 Voltage | Current at Node 2 | Current at Node 3 |
| --- | --- | --- | --- | --- |
| 1.5 V | 5.0 V | 2.5 V | 0.5 mA | 0.5 mA |

Practical Values

| R1 Voltage | R2 Voltage | R3 Voltage | Current at Node 2 | Current at Node 3 |
| --- | --- | --- | --- | --- |
| 1.5 V | 5.0 V | 2.5 V | 0.5 mA | 0.5 mA |

# Post Lab Questions

- Define Energy.

Energy is the capacity to do work or cause change in a physical system. In electrical circuits, energy is the total work done by charges moving through a circuit under the influence of an electric potential difference (voltage). It is measured in Joules (J). Electrical energy is given by: E = P × t = V × I × t, where V is voltage, I is current, and t is time.

- Define Power.

Power is the rate at which energy is consumed or transferred per unit time. In electrical circuits, power represents how quickly electrical energy is converted into other forms (heat, light, mechanical energy). It is measured in Watts (W). Electrical power is calculated as: P = V × I = I² × R = V²/R, where V is voltage, I is current, and R is resistance.

- What is Charge?

Electric charge is a fundamental physical property of matter that causes it to experience a force when placed in an electromagnetic field. It is measured in Coulombs (C). Charge is carried by electrons (negative charge, -1.6 × 10■¹■ C) and protons (positive charge). In a circuit, current is the flow of electric charge: Q = I × t, where Q is charge (C), I is current (A), and t is time (s).

- What is a Circuit?

A circuit is a closed, continuous path through which electric current can flow. It consists of a voltage source (battery or power supply), conducting wires, and one or more components (resistors, capacitors, etc.). A circuit must form a complete loop for current to flow. Circuits can be series (single path), parallel (multiple paths), or a combination of both.

- What is the Role of a Resistor in a Series Circuit?

In a series circuit, a resistor: (1) Limits the total current flowing through the circuit — higher resistance means lower current. (2) Drops a portion of the supply voltage across itself, following Ohm's Law (V = IR). (3) Dissipates electrical energy as heat (P = I²R). (4) In a voltage divider configuration, each resistor drops a voltage proportional to its resistance relative to the total resistance. The total resistance of resistors in series simply adds up: R_total = R1 + R2 + R3 + ...

- What is the Voltage Divider Rule?

The Voltage Divider Rule (VDR) states that in a series circuit, the voltage across any resistor is equal to the supply voltage multiplied by the ratio of that resistor's value to the total series resistance. Formula: V_Rx = V_s × (Rx / R_total). For example, in Task 1: V_R1 = 5V × (5.1 kΩ / 8.4 kΩ) = 3.036 V. This rule is useful for calculating individual voltage drops without first calculating the current explicitly.

- What are the Features of a Series Circuit?

The key features of a series circuit are: (1) Single Current Path — current has only one route to flow through all components. (2) Same Current Everywhere — the current is identical through every element: I_total = I_R1 = I_R2 = I_R3. (3) Voltage Division — the supply voltage splits across components in proportion to their resistance values. (4) Total Resistance is Additive — R_total = R1 + R2 + R3 + ... (5) If one component fails (open circuit), the entire circuit stops working. (6) KVL holds: V_s = V_R1 + V_R2 + V_R3.

- Differentiate Between the Internal Circuit of a Voltmeter and an Ammeter.

Voltmeter: It has a very HIGH internal resistance (ideally infinite). It is connected in PARALLEL with the component under measurement. The high resistance ensures negligible current flows through it, so it does not disturb the original circuit. It measures the potential difference (voltage) across a component.

Ammeter: It has a very LOW internal resistance (ideally zero). It is connected in SERIES with the circuit. The low resistance ensures it causes minimal voltage drop, so it does not significantly alter the circuit current. It measures the current flowing through a branch.

- In Which Combination Do We Connect a DMM to Measure Current and Voltage?

To measure VOLTAGE: The DMM (set to voltmeter mode) is connected in PARALLEL (across) the component or source. This works because the meter's high internal resistance draws negligible current from the circuit.

To measure CURRENT: The DMM (set to ammeter mode) is connected in SERIES with the circuit. The circuit must be broken at the measurement point and the meter inserted so all current flows through it. Never connect the ammeter in parallel — this would cause a short circuit and damage the meter due to its very low internal resistance.