SUMMARY OF STEPS OF LAB 10

focused on the verification of Thevenin’s Theorem, which states that any linear, bilateral, active two-terminal network can be replaced by an equivalent circuit consisting of a single voltage source  in series with a single resistance

Based on the lab manual, here is the summary of the steps taken to perform this experiment:

1. Initial Circuit Setup and Load Current Measurement

- Connections: The initial circuit was constructed on a breadboard as per the provided diagram, featuring three resistors and a  load resistor.

- Measurement: A Regulated Power Supply (RPS) was set to a specific voltage, and the corresponding load current was measured using a milliammeter connected in series with the 1k Omega resistor.

2. Finding Thevenin’s Voltage

- Open Circuiting: To find VTH, the load resistor 1k Omega was removed from the circuit.

- Measurement: The voltage across the resulting open terminals Aand D in the diagram was measured using a digital multimeter. This value is the open-circuit voltage, VOC

3. Finding Thevenin’s Resistance R TH

- Deactivating Sources: The power supply (RPS) was removed and replaced with a short circuit.

- Measurement: The equivalent resistance "looking back" into the open load terminals was measured using a multimeter.

4. Equivalent Circuit Verification

- Setup: An equivalent circuit was built using a power supply set to the measured VTH and a Decade Resistance Box (DRB) set to the measured RTH

- Comparison: The original $1k\Omega$ load resistor was reattached to this new, simplified circuit.

- Verification: The current flowing through the load in this equivalent circuit was measured and compared against the current from the original complex circuit. If the values match, the theorem is verified.

PROTEUS SCREENSHOTS