SUMMARY OF STEPS OF LAB NO 4

Lab 4 — Parallel Circuit Implementation

Vs = 9V (both tasks) ·

Tools:

DMM

Breadboard

power supply

Resistance & voltage measurement

1

Same resistance procedure as Lab 3 — power off, isolate component, use Ohms setting.

2

For voltage, connect DMM in parallel across each branch. All branches show same voltage equal to the supply.

Current measurement (parallel-specific)

1

Turn off power. Break the individual branch whose current you want to measure.

2

Insert DMM in series within that branch only. Set to DC Amps / mA.

3

Turn power on, record the reading. Repeat for each branch. Each branch current = V / R of that branch.

4

Sum of all branch currents = total supply current (KCL verification).

Lab tasks performed

T1

Built parallel circuit (9V, R1 = 10 kΩ, R2 = 2 kΩ, R3 = 1 kΩ). Measured branch currents and voltage between nodes 2 and 7.

T2

Built parallel circuit (9V, R1 = 100 Ω, R2 = 3 kΩ, R3 = 5 kΩ, R4 = 10 kΩ). Measured all four branch currents and voltage between nodes a and e.